### 1. Universal Motor (Final 2023 Q1 — Question Bank Pages 7 & 12 [Mark: 03])

Q1: Define universal motor. Differentiate universal motor from a DC series motor and also mention its applications.

- Definition: A Universal Motor is a specially designed fractional-horsepower series-wound commutator motor that can operate on either a DC supply or a single-phase AC supply at approximately the same speed and power output.

| Feature | Universal Motor (AC/DC Series Motor) | Ordinary DC Series Motor |
| --- | --- | --- |
| 1. Core Lamination | Entire magnetic path (stator yoke, poles, and rotor) is heavily laminated to minimize AC eddy-current and hysteresis losses. | Only the rotor is laminated; stator yoke and poles are often solid steel (would overheat on AC). |
| 2. Field Winding Turns | Has fewer field turns (with a low-reluctance, larger pole area) to keep AC inductive reactance drop (IXf) low and power factor high. | Has more field turns (which would cause a huge IXf voltage drop and poor power factor on AC). |
| 3. Armature & Compensating Windings | Has more armature conductors (to maintain torque with fewer field turns) and often includes compensating windings to cancel AC reactance sparking. | Standard armature turns; rarely uses compensating windings in small ratings. |
| 4. Brushes | Uses high-resistance carbon brushes to suppress severe AC transformer-EMF sparking in shorted coils during commutation. | Uses lower-resistance carbon/graphite brushes. |

### 2. Stepper Motor (Final 2024 Q0 & Final 2022 Q3 — Question Bank Page 12 [Marks: 03])

Q2: What is a stepper motor? Briefly describe its operation and applications. Draw the truth table and corresponding applied voltage waveforms to make the stepper motor step 30∘.

- Definition: A Stepper Motor is a brushless, synchronous electromechanical device that converts digital electrical voltage pulses into fixed, discrete mechanical angular steps of its rotor shaft (used for precise open-loop position and speed control).

- Brief Operation (3 Lines):

  - The stator contains multipole phase windings (a,b,c) excited sequentially by an electronic DC pulse driver, while the rotor is either a permanent magnet or a toothed soft-iron core (variable reluctance).

  - By switching the DC pulses from one stator phase to the next in sequence, the stator magnetic field steps around the air gap, causing the rotor to rotate in exact angular increments (θstep=2Pθe).

- Applications:

  - CNC Machines, 3D Printers, and Laser Cutters (precise X-Y-Z tool positioning).

- Truth Table & Waveforms for a 30∘ Mechanical Step (θm=30∘): For a standard 3-phase stator (a,b,c) with a 4-pole rotor (θm=2Pθe=12θe, where each 60∘ electrical step produces a 30∘ mechanical step in full-step mode; or for a 2-pole rotor in half-step mode):

| Pulse / Step # | Phase a Voltage (Va) | Phase b Voltage (Vb) | Phase c Voltage (Vc) | Electrical Angle (θe) | Mechanical Rotor Angle (θm, 4-Pole) |
| --- | --- | --- | --- | --- | --- |
| 1 | +V | 0 | 0 | 0∘ | 0∘ |
| 2 | 0 | 0 | −V | 60∘ | 30∘ |
| 3 | 0 | +V | 0 | 120∘ | 60∘ |
| 4 | −V | 0 | 0 | 180∘ | 90∘ |
| 5 | 0 | 0 | +V | 240∘ | 120∘ |
| 6 | 0 | −V | 0 | 300∘ | 150∘ |
| 7 | +V | 0 | 0 | 360∘ | 180∘ |

- (Note: If your class used a 2-pole rotor (θm=θe) to get 30∘ steps via Half-Stepping, simply insert the 2-phase-ON intermediate rows between the rows above: e.g., at 30∘: Va=+V,Vb=0,Vc=−V; at 90∘: Va=0,Vb=+V,Vc=−V, etc.!)

- How to Sketch the Applied Voltage Waveforms (Va,Vb,Vc vs. Time/Steps): Draw 3 horizontal time axes stacked vertically for Va(t), Vb(t), and Vc(t) marked with steps 1,2,3,4,5,6:

  - On Va: Draw a rectangular pulse of height +V during Step 1, 0 for Steps 2–3, and −V during Step 4.

  - On Vb: Draw +V during Step 3, and −V during Step 6 (0 elsewhere).

  - On Vc: Draw −V during Step 2, and +V during Step 5 (0 elsewhere).

### 3. Capacitor-Start and Run Motor (Final 2023 Q2 — Question Bank Page 12 [Mark: 02])

Q3: Explain the functions of the capacitor in a capacitor-start and run motor.

- Exam Answer (3 Crisp Points):

  - Makes the Motor Self-Starting (Phase Splitting): A single-phase winding alone creates only a pulsating magnetic field (0 starting torque). Connecting a capacitor in series with the auxiliary winding makes the auxiliary current (Iaux) lead the main winding current (Imain) by nearly 90∘, creating a two-phase rotating magnetic field that makes the motor self-starting with high starting torque.

  - Improves Running Power Factor & Efficiency: Keeping the run capacitor permanently in series with the auxiliary winding during normal running supplies reactive magnetizing current locally, which raises the motor's running power factor (close to unity) and increases full-load efficiency.

### 4. Bonus: The Two 2019 AC Fundamental Questions at the Bottom of Page 12

Just so 100% of Page 12 of your Question Bank is complete:

- Final 2019 Q1 [Mark: 03] (Electrical vs. Mechanical Degree & Frequency):

  - In a P-pole AC machine, one complete cycle of induced voltage (360∘ electrical) occurs every time a pair of poles passes a conductor (2P of a mechanical revolution). Thus:

θe=P2θm

  - At rotor speed nm rpm (nm60 rev/s), since there are P2 electrical cycles per mechanical revolution, the electrical frequency in Hz (cycles/s) is:

f=P2 cycles/rev×nm60 rev/s=Pnm120 Hz

- Final 2019 Q2 [Mark: 04] (P=4 poles, 3-phase double-layer winding, S=24 slots, ϕ=0.022 Wb, nm=1800 rpm):

  - Slot angle γ=180∘PS=180∘×424=30∘; Slots/pole/phase n=S3P=243×4=2.

  - (i) Distribution Factor (Kd):

Kd=sin(nγ/2)nsin(γ/2)=sin(2×30∘/2)2sin(30∘/2)=sin 30∘2sin 15∘=0.966

  - (ii) Generated Voltage per Phase (Eph): Frequency f=Pnm120=4×1800120=60 Hz. With 24 coils total (8 coils/phase), for Nc-turn full-pitch coils (Tph=8Nc, Kp=1):

Eph=4.44KpKdfϕTph=4.44×1×0.966×60×0.022×(8Nc)=45.28Nc Volts (or 45.28 V for Nc=1)